Sunday,18,June,2017,22:16:06:64556

VCDS -- Windows Based VAG/VAS Emulator Running on Windows 10 x64

VCDS Version: 17.1.3.0 (x64)

Data version: 20170320 DS267.7

www.Ross-Tech.com

VIN:    License Plate:

--------------------------------------------------------------------------------------------------------------------------------------------

Chassis Type: 8E - Audi A4/S4/RS4 B6/B7 (2001 > 2008)

Scan: 01 02 03 08 09 0F 11 15 16 17 18 25 36 37 45 46 55 56 57 65

67 69 75 76 77

----------------------------------------------------------------------

Address 08: Auto HVAC        Labels: 8E0-820-043.lbl

Part No: 8E0 820 043 AH

Component:   A4 Klimaautomat   1424

Coding: 00001

Shop #: WSC 06325

VCID: 67D3B1860591F87948-5178

3 Faults Found:

01809 - Actuating Motor for Temperature Flap; Left (V158)

41-10 - Blocked or No Voltage - Intermittent

01273 - Fresh Air Blower (V2)

30-10 - Open or Short to Plus - Intermittent

01273 - Fresh Air Blower (V2)

29-10 - Short to Ground - Intermittent

These are new. I've been trying to diagnose / test for faulty Fuel Pump and Relay since Friday (so at least 2 days of intermittent testing) without the battery having had a chance to recharge - although a quick test shows 12.1v (yesterday was 13.4). Could these be caused by low voltage? Tomorrow night I'll check the passenger foot-well for signs of any water getting into the Comfort Controller Unit.

----------------------------------------------------------------------

Address 36: Seat Mem. Drvr        Labels: 8E0-959-760.lbl

Part No: 8E0 959 760

Component: Sitzmemory R2 F     0305

Coding: 00002

Shop #: WSC 06325

VCID: 71E7DFDE3B5536C916-4B18

1 Fault Found:

00668 - Supply Voltage Terminal 30

07-10 - Signal too Low - Intermittent

Could this also be caused by low voltage due to testing Fuel Pump / Fuel Relay draining battery down to 12.1v?

End-------------------------(Elapsed Time: 05:46)---------------------

----------------------------------------------------------------------

----------------------------------------------------------------------

Chassis Type: 8E (8E - Audi A4/S4/RS4 B6/B7 (2001 > 2008))

Scan: 01 02 03 08 09 0F 11 15 16 17 18 25 36 37 45 46 55 56 57 65

67 69 75 76 77

VIN: WAUZZZ8E15A115366   Mileage: 206540km-128338miles

----------------------------------------------------------------------

Address 01: Engine        Labels: 079-910-560-BBK.lbl

Part No SW: 8E0 910 560 B    HW: 8E0 907 560

Component: 4.2L V8/5V          0020

Coding: 0011771

Shop #: WSC 06325 000 00000

VCID: 2449E68AE427D361AD-515A

4 Faults Found:

16706 - Engine Speed Sensor (G28)

P0322 - 004 - No Signal - Intermittent

18308 - Coolant Fan Control 2

P1900 - 004 - Open or Short to Ground - Intermittent

18010 - Power Supply Terminal 30

P1602 - 002 - Voltage too Low - Intermittent

17908 - Fuel Pump Relay (J17)

P1500 - 004 - Electrical Malfunction - Intermittent

Readiness: 0000 0000

These 4 are all new.

----------------------------------------------------------------------

Address 02: Auto Trans        Labels: 09L-927-760.lbl

Part No SW: 8E0 910 156 R    HW: 09L 927 156

Component: AG6 09L 4.2L5V RdW  0060

Revision: --H01---    Serial number:        0059020

Coding: 0000001

Shop #: WSC 06325 000 00000

VCID: 356F2BCE5FBD5AE90A-5178

1 Fault Found:

16944 - System Voltage

P0560 - 000 - Malfunction - Intermittent

Freeze Frame:

Fault Status: 00110000

Fault Priority: 0

Fault Frequency: 1

Reset counter: 40

Mileage: 206540 km

Time Indication: 0

Freeze Frame:

RPM: 0 /min

RPM: 0 /min

RPM: 0 /min

(no units): 30.0

(no units): 85.0

Temperature: 14.0°C

T.B. Angle: 0.0°

These are all new.

----------------------------------------------------------------------

Address 03: ABS Brakes        Labels: 8E0-614-517.lbl

Part No: 8E0 614 517 Q

Component: ABS/ESP allrad      4533

Coding: 04597

Shop #: WSC 00010

VCID: 366D2CC212BB51F103-513C

1 Fault Found:

01826 - Sensor for Steering Angle (G85); Supply Voltage Terminal 30

35-00 -  -

This is new

----------------------------------------------------------------------

Address 08: Auto HVAC        Labels: 8E0-820-043.lbl

Part No: 8E0 820 043 AH

Component:   A4 Klimaautomat   1424

Coding: 00001

Shop #: WSC 06325

VCID: 67D3B1860591F87948-515A

4 Faults Found:

01809 - Actuating Motor for Temperature Flap; Left (V158)

41-10 - Blocked or No Voltage - Intermittent

01273 - Fresh Air Blower (V2)

30-00 - Open or Short to Plus

01273 - Fresh Air Blower (V2)

29-10 - Short to Ground - Intermittent

01206 - Signal for Duration of Ignition Off Time

27-00 - Implausible Signal

This is new

----------------------------------------------------------------------

Address 09: Cent. Elect.        Labels: 8E0-907-279-8E2.lbl

Part No: 8E0 907 279 F

Component: int. Lastmodul  RDW 0517

Coding: 01011

Shop #: WSC 06325

VCID: 356F2BCE0FBD5AE90A-4B32

1 Fault Found:

00987 - Lamp for Brake Light; Left (M9)

57-00 - Electric Circuit Failure

Rear bulb needs replacing (intermittent fault - bulb on its way out).

----------------------------------------------------------------------

Address 15: Airbags        Labels: 8E0-959-655-84.lbl

Part No: 8E0 959 655 J

Component: Airbag 8.4EP        1002

Coding: 0010702

Shop #: WSC 06325 000 00000

VCID: 3E7D34E26A8B99B1DB-5178

No fault code found.

----------------------------------------------------------------------

Address 16: Steering wheel        Labels: 8E0-953-549.lbl

Part No: 8E0 953 549 N

Component: Lenksáulenmodul     0611

Coding: 01112

Shop #: WSC 06325

VCID: 3E7D34E23A8B99B1DB-4B32

No fault code found.

----------------------------------------------------------------------

Address 17: Instruments        Labels: 8E0-920-9xx-8E2.lbl

Part No: 8E0 920 931 A

Component: KOMBI+WEGFAHRS. RB4 D42

Coding: 03501

Shop #: WSC 00001

VCID: 264DFC82C21B2171B3-5178

WAUZZZ8E15A115366     AUZ6Z0D0186020

2 Faults Found:

00771 - Fuel Level Sensor (G)

30-00 - Open or Short to Plus Have removed the fuel pump and disconnected power to the cap so this is just an open circuit.

01300 - Control Module for Navigation with CD-Rom (J401)

49-10 - No Communications - Intermittent I removed the CD Controller ages ago (along with the glove compartment - so my dog could fit in the front foot-well comfortably without being squashed :)

----------------------------------------------------------------------

Address 36: Seat Mem. Drvr        Labels: 8E0-959-760.lbl

Part No: 8E0 959 760

Component: Sitzmemory R2 F     0305

Coding: 00002

Shop #: WSC 06325

VCID: 71E7DFDE3B5536C916-4B32

1 Fault Found:

00668 - Supply Voltage Terminal 30

07-10 - Signal too Low - Intermittent Again - battery getting low? All of the driver's seat memory functions work fine. The only thing I've noticed is that "sometimes" the passenger side-mirror doesn't return to quite the right position after reversing. It's been like this for a while. Hitting the memory button again usually returns it. Only happens maybe once every 20 or so times.

----------------------------------------------------------------------

Address 37: Navigation        Labels: None

Part No: 8E0 035 192 A

Component: TV/Navigation   H09 0209

Shop #: WSC 00010 824 83250

VCID: 2245E092FE33DD51DF-513C

1 Fault Found:

01065 - Connection from RNS to TV Antenna Amplifier

012 - Electrical Fault in Circuit This is new - that said, last ran VCDS on v15.x - have just upgraded today to v17 - I don't think this was in v15 (can't find reference in the log files to "Address 37" or "Navigation") so the fault may have been there but just not been reported??

----------------------------------------------------------------------

Address 45: Inter. Monitor        Labels: 8E0-951-177.lbl

Part No: 8E0 951 177

Component: Innenraumueberw.   0804

Coding: 00001

Shop #: WSC 00000

VCID: 6BDBCDB61979DC192C-5178

No fault code found.

----------------------------------------------------------------------

Address 46: Central Conv.        Labels: 8E0-959-433-MAX.clb

Part No: 8E0 959 433 BB

Component: Komfortgerát T7B    1612

Coding: 07114

Shop #: WSC 07130

VCID: 72E5D0D23E530DD1EF-4B32

Subsystem 1 - Part No: 8E2959802

Component: Tõrsteuer.FS BRM 0205

Subsystem 2 - Part No: 8E2959801

Component: Tõrsteuer.BF BRM 0205

Subsystem 3 - Part No: 8E0959801A

Component: Tõrsteuer.HL BRM 0205

Subsystem 4 - Part No: 8E0959802A

Component: Tõrsteuer.HR BRM 0205

5 Faults Found:

01370 - Alarm triggered by Interior Monitoring

35-00 -  -

00962 - Alarm via. Tilt Sensor

35-00 -  -

01368 - Alarm triggered by Luggage Compartment Switch

35-00 -  -

01371 - Alarm triggered by Door Contact Switch; Driver's Side

35-00 -  -

00956 - Key 2

35-00 -  -

I suspect these are all as a result of disconnecting the power in between testing of the Fuel Pump and FUel Pump Relay. When I'm testing the pump / relay I'm just doing a 3/4 turn to prime the pump - I'm not trying to crank the engine. I know that each time I reconnect the power the central locking doesn't work from the remote key-fob - I think it only re-partners with the key-fob once FULL ignition is started to crank the engine?

----------------------------------------------------------------------

Address 55: Headlight Range        Labels: 4Z7-907-357.lbl

Part No: 4Z7 907 357

Component: dynamische LWR       D09

Coding: 00006

Shop #: WSC 06325

VCID: 0409860A14E73361CD-4B32

1 Fault Found:

01539 - Headlights Not Adjusted

35-00 -  -  For a while now the car has been telling me there's a problem with the auto-level motor - but it seems to be working just fine. Has done for the past year and a half.

----------------------------------------------------------------------Address 56: Radio        Labels: None

Part No: 8E0 035 192 A

Component: Radio           H09 0205

Coding: 0001047

Shop #: WSC 00010 824 83250

VCID: 2245E092FE33DD51DF-513C

2 Faults Found:

00821 - Antenna 2 for Radio (R93)

011 - Open Circuit

00857 - CD Changer Unit (R41)

004 - No Signal/Communication see previous comment re. removal of glove compartment to accomodate the Executive Pooch.

----------------------------------------------------------------------

Address 76: Park Assist        Labels: 8E0-919-283-8E2.lbl

Part No: 8E0 919 283 A

Component: Parkingsyst. A4 RDW D08

Coding: 01114

Shop #: WSC 06325

VCID: 2E5D04A2EAEB69314B-515A

3 Faults Found:

00625 - Vehicle Speed Signal

27-10 - Implausible Signal - Intermittent This is new.

01545 - Sensor for Parking Aid; Rear Left (G203)

37-00 - Faulty Known fault. Fortunately I'm gifted at parking and don't need the sensors :)

01546 - Sensor for Parking Aid; Rear Mid-Left (G204)

37-00 - Faulty

End-------------------------(Elapsed Time: 05:13)---------------------